U-Boot – Build and Debugging Setup on Softconsole

1. Get the RISC-V U-Boot source from the githup.

$ git clone https://github.com/RISCV-on-Microsemi-FPGA/RISC-V_U-Boot

$ cd RISC-V_U-Boot

2. Open Softconsole and create a workspace for building and debugging U-Boot.

3.  Create a project – Select File -> New -> C Project

Write the project name, select RISC-V Embedded Application Empty Project and click on Finish.

4. Import U-Boot sources into the project – Select File -> Import -> General -> File system

Press next.

Browse for RISC-V U-Boot source directory on local machine and select all files and Click on Finish.

5. Set the Build process – Right click on Project -> Properties -> C/C++ Build

C/C++ Build -> Builder Settings

- Uncheck “Generate Makefiles Automatically”

- Build directory to the project, remove 'Debug'

Click on Apply.

C/C++ Build -> Behavior settings

- Uncheck “Build on resource save”

- Clear “Build(incremental build)”

- Write 'distclean'  on “Clean”

Click on Apply.

5.1. Set the RISC-V cross tools - C/C++ Build -> Environment -> add (twice)

Name : CROSS_COMPILE

Value : riscv32-unknown-linux-gnu-

Click on OK

Name: PATH

Value : [risc-v cross tool installation directory]/bin:$PATH

Click on OK and Apply... OK.

5.2. Create Make Targets – Right Click on Project -> Make Targets -> Create (twice)

Target Name: distclean

Target Name : riscv-m2sxxx_defconfig

Click on OK

5.3. Build U-Boot

- Set macro CONFIG_SYS_TEXT_BASE to SDRAM base address(0x80000000) in include/configs/riscv-m2sxxx.h

- Right Click on Project -> Clean Project

- Right Click on Project -> Make Targets -> Build

Select 'riscv-m2sxxx_defconfig' and press Build

- Right Click on Project -> Build Project

- The u-boot output files available in project root directory(u-boot.bin, u-boot...etc)

6. U-Boot Debugging Setup

- Right Click on Project -> Debug As -> Debug Configurations

On GDB OpenOCD Debugging - Right Click and select New

GDB OpenOCD Debugging -> Main

C/C++ Application : u-boot

GDB OpenOCD Debugging -> Debugger

Edit GDB Client Setup section

- Executable : [risc-v cross tool installation directory]/bin/riscv32-unknown-linux-gnu-gdb

- Other options : u-boot

GDB OpenOCD Debugging -> Startup

Set breakpoint at : board_init_f

Click on Apply and to start debugging Press “Debug”.

Before Pressing “Debug” - SmartFusion2 M2S150 Advance Dev Kit is powered ON and FlashPro5 and USB-serial cable is connected between the Board and PC. USB-serial baud rate is 115200.

Run the U-Boot and see the messages on serial terminal.

7. U-Boot Run From eNVM

- Set macro CONFIG_SYS_TEXT_BASE to eNVM base address(0x60000000) in include/configs/riscv-m2sxxx.h

- Right Click on Project -> Clean Project

- Right Click on Project -> Make Targets -> Build

Select 'riscv-m2sxxx_defconfig' and press Build

- Right Click on Project -> Build Project

- The u-boot output files available in project root directory(u-boot.bin, u-boot-nodtb.bin...etc).

- Generate ihex file from binary file of u-boot.

$ riscv32-unknown-linux-gnu-objcopy -I binary -O ihex u-boot-nodtb.bin u-boot.ihex

- Change u-boot.ihex to u-boot.ihx

- Upload u-boot.ihx file in eNVM using Libero design(available on RISC-V-on-Microsemi-FPGA Github) and  generate staple file.

- Program staple file on Smartfusion2 M2S150 Adv Dev Kit using FlashPro5.

- On reset, the serial terminal displays the u-boot messages.